Technical aids supporting computer use by the blind and sight impaired: How tests can improve the quality of available products

Brigitte Bornemann-Jeske / Thomas Lilienthal

BIT GmbH / DIAS GmbH, Hamburg

(Paper accepted for Presentation at the AAATE Conference Nov. 1999 at Dortmund)

Computer aids, blind and sight-impaired computer users, test procedures

Abstract:

A procedure for testing the usability of computer aids for blind and sight-impaired PC users will be presented. Comparative product tests evaluate the usability of important computer aids for completing typical office work. The procedure was developed as part of the BITE Project, then modified and tested on Braille bars, screen readers for Windows and screen magnifying systems. The data recorded offer experts, trainers and affected users the necessary basis for making decision about the selection and purchase of appropriate aids. Suppliers received important information for future product development. We plan to make the test procedures and results available in an Internet databank in the future.

1. Information technology as the key to the vocational integration of blind and sight-impaired workers

Communication technology is of central importance to the vocational integration of blind and sight-impaired people. The reasons for this include the disappearance of "classic" jobs for the blind such as telephone operators and typists, as well as the increasing combination of qualified jobs and new fields (such as call centres) with the computer. The ability to efficiently use computers and the Internet is becoming ever more decisive for the personal effectiveness of severely disabled workers and for their competitiveness on the labour market.

The blind and sight impaired depend upon appropriate technical aids in order to be able to work with computers. The aids' operational capabilities as well as the workers' ability to use these aids are of decisive importance for the vocational integration of this group of people. However, computer aids are not always able to live up to their expectations. The domination of graphical user interfaces has made it difficult to develop aids that make it possible to work effectively with current Windows applications.

This, in turn, has made it difficult for users and advisers to select a suitable computer aid, and the purchase of inappropriate articles is not an uncommon event. This is partly due to the heterogeneous spectrum of products available, but it also has to do with a lack of uniform quality standards and objective information about the performance of the aids. Sometimes even the suppliers of Windows adaptions themselves do not know for which applications their product is suitable.

2. The BITE Project:  using product tests to provide information about product performance and to stimulate product innovation

To solve the above mentioned problems associated with computer aids which are so important to the vocational rehabilitation of the blind and sight impaired, the firms BIT GmbH and DIAS GmbH in Hamburg were commissioned in 1996 to lead the BITE Project. The goal of the project sponsored by the German Federal Ministry of Labour and Social Affairs and by the Central Social Services of Hamburg (Hauptfürsorgestelle) was to develop and test an appropriate procedure for evaluating important computer aids for the blind and sight impaired. The results should help users, experts and advisers select the most appropriate solution and, at the same time, motivate suppliers to improve their products.

Corresponding test procedures were developed for the product groups Braille bars, Windows adaptions, speech output and screen magnifiers. These test procedures were verified in connection with product tests. At this point we do not wish to discuss the test results [1], but would rather report on the underlying test procedure.

The original concept: comparative product tests under laboratory conditions

Like all other hard- and software, computer aids are used in complex contexts. These contexts are broken down in the norm DIN/EN 29421 (Evaluating the usability of hard- and software according to user needs, tasks and work environment) [2]. The other strategy of this norm (differentiating between product quality according to effectiveness, efficiency and satisfaction by goal attainment) is also applicable to computer aids [3].

Users

Computer equipment

Computer aids

Work environment

Tasks

Goals

desired work results

Figure 1: Computer Aids

Requirement criteria

Following DIN/EN 29421, requirement criteria for different aids were developed in cooperation with experts and users. Three areas can be differentiated. Compatibility and operational security of the aids as well as extent and quality of the services offered and the accompanying product information make up the first of these areas. At the centre of the requirement criteria we find the tasks that must be completed by blind and sight-impaired computer users with the help of technical aids. The central question addressed here was whether the most important applications become sufficiently accessible with the help of the aids. Efficiency and ergonomics of the computer aids make up the focus of the third area. Ease of differentiation between screen contents and ease of use of the control elements are required[1].

The test procedure

The requirement criteria was made operational with the use of different test procedures. In order to test whether the "general standards of product quality for hard- and software" were adhered  to, surveys of suppliers and analyses of documentation were  conducted.  The most

| Requirement criteria | Test procedure |
| --- | --- |
| 1. General standards of product quality for hard- and software are adhered to |  |
| 1.1. Compatibility |  |
| 1.2. Operational security | Survey of suppliers and |
| 1.3. Product information | documentation analyses |
| 1.4. Service |  |
| 2. Functionality according to tasks |  |
| 2.1. Breadth of application |  |
| 2.2. Basic functions of standard software operational in an efficient and satisfactory way | Laboratory tests |
| 2.3. Advanced functions for secretaries and executives operationally efficient |  |
| 2.4. For users with little support by operational service, special functions should be operational |  |
| 3 Efficient access to information |  |
| 3.1. All components of the screen information are rendered completely and such that they are easy to differentiate from one another. | Laboratory tests |
| 3.2. Operational control |  |
| 3.3. Individualisation | No appropriate test procedure |
| 3.4. Ease of learning | existing |

Figure 2: Requirement Criteria

extensive test was the examination of all product groups in the area of functionality according to tasks. In order to test the accessibility of software applications, a catalogue of model tasks was made up and given to blind computer users as part of laboratory tests. The evaluation of "efficient access to information" was limited to basic aspects of information presentation. The tests for individualisation and ease of learning could not be conducted due to the lack of an operational basis for software ergonomics in the area of computer aids.

##### Positive experiences and the need for improvement

The procedure for laboratory tests was developed and executed in 1997 and 1998 based on the product groups Braille bars and Windows adaptions for the blind. The product-group specific requirement criteria, developed in cooperation with experts and users, proved to be worthwhile. The requirement criteria also proved to be appropriate for the necessary adaptation to technical development and the changing demands of the workplace.

Experiences with the test procedure were mixed. The laboratory tests for Braille bars on the DOS system were acceptable, but those for screen readers for Windows were less acceptable. The cooperation of the system components, access aids, PC hardware, operating system and software application under MS-Windows proved to be much more complicated than under MS-DOS, and innovation cycles are much shorter for Windows. The capabilities of screen readers are still deficient, and the products offer very different solution strategies. This type of situation requires extensive, time-consuming test and evaluation procedures that can barely keep up with product development. A comparative test for all screen readers available had been completed once, but a continual adherence to the procedure of comparative lab tests could not be justified due to the extreme costs and the fact that the results become outdated so quickly.

##### How to evaluate results?

The original procedure also had weaknesses in regard to the evaluation and presentation of test results. The user panel that participated in the project rejected the originally planned rating of the test results. Users fear that the individual selection of computer aids that is so important due to the different capabilities and the heterogeneous solutions employed by individual computer aids could be endangered by a rating of individual products. Users feel that detailed and up-to-date descriptions of the capabilities of computer aids are what is needed. With the help of this data, the most appropriate product for each individual work situation should be selected. This suggestion was integrated in the evaluation process. Users also made suggestions regarding the modification of the test procedure [4] that, together with experiences gathered during the project, make up the foundation for the current test procedure.

3. The modified concept: involving suppliers, experts and users in the test procedures

In order to solve the problems described, a modified procedure was tested during the development of the test procedure for the product group screen magnifiers. Following standard procedures for self certification, e.g. DIN ISO 9000, the most important components of the available collection instruments were standardised as far as possible so that they became usable even outside the laboratory. The procedure allows product suppliers as well as institutions involved in advising and vocational training and, last but not least, user experts to conduct independent tests.

The external use of the test procedure makes high demands on the comprehensibility and clearness of the collection instruments. In addition to unambiguous standards, precisely defined measuring procedures are the main requirement. The advantages of such a test programme are as follows:

##### Product tests can be conducted by suppliers, experts and users.

- Product suppliers can use the test procedures for internal product development.

- Uniform test criteria and measuring procedures will make it possible to establish binding performance standards.

##### The modulated design of the instrument will make it easier to update and expand the test procedure.

##### 4. Establishing a pool of information about computer aids for the blind and sight impaired

In order to make up for the lack of information about computer aids for the blind and sight impaired, it is not sufficient just to develop appropriate testing procedures and to publish test results in scientific journals. Fundamental to improving the situation is the practical use and accessibility of such performance descriptions during advising on the issue of computer aids.

One important step is the establishment of a suitable Internet database that would give users, experts, advisers and suppliers access to current test procedures and results. The Internet database would also be a communications forum that allows users and experts to exchange information in the form of experience reports and criticism.

INTERNET-INFOPOOL

Test instruments

Test results

Criticism

Experience reports

Users

Advisers

Product suppliers

Trainers

Figure 3: Internet Information Pool

5. Conclusion

Blind and visually impaired workers using computers as efficiently as sighted people is currently more vision than reality. But on our way to reaching this vision, computer aids for the blind and sight impaired already play a central role in the vocational integration of these groups. Product tests and operational comparisons, as well as presentations and discussions that make use of modern communications technology, are suitable and required for eliminating the confusion and insecurity that surround the selection of appropriate aids. The test results also provide the foundation for the development of product standards which are required because of the various performance levels of products in this field.

References

[1] An extensive presentation of the test results, required criteria and test procedures can be found, in German, for the product groups Braille bars and Windows adaptions for the blind under: Modellvorhaben BITE, Zwischenbericht 1997, Hamburg 1998, and for screen magnifiers under: Modellvorhaben BITE, Abschlußbericht 1999, Hamburg 1999. The reports can be ordered in printed form from BIT GmbH, and can be downloaded from the server of the firm DIAS GmbH (ftp://gate.dias.de/pub/bite). The reports are included in the information system for vocational rehabilitation, REHADAT, Research Database, at http://www.rehadat.de

[2]

DIN EN 29241: Deutsches Institut für Normung: Ergonomische Anforderungen für Bürotätigkeiten mit Bildschirmgeräten (= ISO 9241: Ergonomic requirements for office work with visual display terminals), Beuth Verlag, Berlin 1993

DIN EN 29241 Teil 10: Grundsätze der Dialoggestaltung (= ISO 9241 part 10: Dialog principles), Entwurf, Beuth Verlag, Berlin 1993

DIN EN ISO 9241 Teil 11: Anforderungen an die Gebrauchstauglichkeit - Leitsätze (Guidance on Usability), Entwurf, Beuth Verlag, Berlin 1995

[3] See also Wolfgang.Wünschmann: Gebrauchstauglichkeit technischer Hilfen für Menschen mit besonderen Bedürfnissen, in: Bundesanstalt für Arbeitsschutz und Arbeitsmedizin (Hrsg): Behinderte in der Arbeitswelt, Tagungsbericht der Kongreßreihe, 21. und 22. Mai 1997, Dortmund, Dortmund 1998

[4] Rüdiger Leidner: Computerhilfen für Blinde und Sehbehinderte sinnvoll testen. Vorschläge für einen praxisgerechten Kriterienkatalog, in: Sozialrecht + Praxis, Nr. 9/1996, S. 601-604